Activity 18									          AS 18.1

Student Activity Sheet

Electronic Bill of Materials

#### Name ________________________

Student Objectives:

1.	Develop a bill of materials using a spreadsheet program.

2.	Describe the advantages of using a spreadsheet in developing a bill of materials.

Equipment and Materials:

- Computer

- Spreadsheet program (e.g., Excel, Lotus 1-2-3)

- Bill of materials_template (Excel file)

- SG 18.1 Electronic Bill of Materials Scoring Guide

- Floppy disk (one for each student)

- Printer

Procedure:

- Refer to SG 18.1 for the criteria you will be graded on.

- Read scenario 1 below.

Your grandmother loves watching the birds in her back yard.  For her birthday you decide to make her a bluebird house.

3.	Review the bluebird house plan at the end of this procedure and develop a list of the cuts of wood you will need for the project. Record the information below.

Cut list:

__________________________________________________________

__________________________________________________________

__________________________________________________________

__________________________________________________________

__________________________________________________________

__________________________________________________________

4.	Convert these into a purchasing bill of materials using the most cost-effective size of lumber available (i.e., size that will produce the least amount of waste). You decide to use pine. Record the information on the next page. The hardware is listed for you.

Note: Lumber from retailers most commonly starts at 8 feet in length and increases in 2-foot increments and is priced per board foot.

Purchasing bill of materials:

20 - 6p (six-penny) finish nails

1 - 1 ½" screw

5.	Use the bill of materials_template provided on the CD-ROM or start with a new file in a spreadsheet program and enter the purchasing bill of materials for this project. See Figure 18.1 for an example format for a bill of materials for another project.

| Bill of Materials |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name: Kevin Matthews |  |  |  |  | Date: 1/10/2001 |  |  |  |
| Class: Agricultural Structures |  |  |  |  | Project: Sawhorse |  |  |  |
| Part Name | # of Pieces | Size Thick | Width | Length | Type of Material | Price/ Unit | Price/Ft | Cost |
| Side rail | 1 | 2" | 4" | 12 | White pine |  | $0.70 | $8.40 |
| Side rail | 1 | 1" | 6" | 8 | White pine |  | $0.45 | $3.60 |
| Screws | 8 |  |  | 3" | Wood screw | $0.01 |  | $0.08 |
| Screws | 12 |  |  | 1 ½" | Wood screw | $0.02 |  | $0.24 |
| Total cost |  |  |  |  |  |  |  | $12.32 |

Figure 18.1 - Sample bill of materials format

6.	Use the following prices from the local lumberyard for the lumber and hardware for the bluebird house.

| Material | Price/ft | Material | Price/unit |
| --- | --- | --- | --- |
| 2" x 4", 8 feet long | $0.53 | 6p finish nails | $0.01 |
| 2" x 6", 8 feet long | $0.72 | 1 ½" screw | $0.02 |
| 1" x 6", 5 feet long | $0.25 |  |  |
| 1" x 6", 6 feet long | $0.36 |  |  |
| 1" x 4", 5 feet long | $0.20 |  |  |

Activity 18								   AS 18.1 (continued)

7.	Initiate the formula function in your program and enter formulas to total the cost for each item and the cost of all items. See an example formula below for another project.

|  | A | B |  | C | D | E | F | G | H | I |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Bill of Materials |  |  |  |  |  |  |  |  |  |
| 2 | Name: Kevin Matthews |  |  |  |  | Date: 1/10/2001 |  |  |  |  |
| 3 | Class: Agricultural Structures |  |  |  |  | Project: Sawhorse |  |  |  |  |
| 4 | Part Name |  | # of Pieces | Size Thick | Width | Length | Type of Material | Price/ unit | Price/ ft | Cost |
| 5 | Side rail |  | 1 | 2" | 4" | 12 | White pine |  | $0.70 | $8.40 |
| 6 | Side rail |  | 1 | 1" | 6" | 8 | White pine |  | $0.45 | $3.60 |
| 7 Cell I5 contains the following formula to calculate the cost for the side rail: E5*H5 | Screws |  | 8 |  |  | 3" | Wood screw | $0.01 |  | $0.08 |
| 8 | Screws |  | 12 |  |  | 1 ½" | Wood screw | $0.02 |  | $0.24 |
| 9 | Total cost |  |  |  |  |  |  |  |  | $12.32 |

8.	When the bill of materials is complete, save the file as “bill of materials_birdhouse” on a floppy disk or a location your instructor specifies. Ensure your name is on the bill of materials and print the file.

9.	Read scenario 2 below.

Your grandmother loved the birdhouse and now she would like you to make a step stool that she can climb to hang or clean the birdhouse.  Review the step stool plan at the end of this procedure and develop a list of the cuts of wood you will need for the project and the purchasing bill of materials. You decide to use pine. Record the information below. The hardware is listed for you.

Cut list:

__________________________________________________________

__________________________________________________________

__________________________________________________________

__________________________________________________________

__________________________________________________________

__________________________________________________________

Purchasing bill of materials:

16 - 1 ½" sheet-rock screws

10.	For this project, call the local hardware store to determine the prices for the materials you will need.

11.	Open the bill of materials_birdhouse file and save the file as “bill of materials_step stool.”

12.	Enter the purchasing bill of materials information for the step stool into the spreadsheet. Note how the totals are calculated instantaneously.

13.	When the bill of materials is complete, save and print the file. Ensure your name is on the bill of materials.

14.	Read scenario 3 below.

Your grandmother has one last request from you.  She would like you to make a bench where she can sit to watch the bluebirds in her back yard.  You decide to make the following park bench plan.  Develop a list of the cuts of wood and metal you will need for the project and the purchasing bill of materials and list them below. For this project, you will use 16-gauge metal pipe for the legs, seat brace, and center support. The hardware is listed for you.

Cut List:

__________________________________________________________

__________________________________________________________

__________________________________________________________

__________________________________________________________

__________________________________________________________

__________________________________________________________

__________________________________________________________

Purchasing bill of materials:

26 - 2" trailer floor screws

8" x 1/8" x 2" flat metal

10" x 1/8" x 1 ¼" flat metal

15.	Again for this project, call the local hardware store to determine the prices for the materials you will need.

Activity 18								   AS 18.1 (continued)

16.	Open the bill of materials_birdhouse file and save the file as “bill of materials_park bench.”

17.	Enter the purchasing bill of materials information for the park bench into the spreadsheet. Note how the totals are calculated instantaneously.

18.	When the bill of materials is complete, save and print the file. Ensure your name is on the bill of materials.

Drawing reprinted from Single Sheet Agricultural Mechanics Plans. University of Missouri-Columbia: Instructional Materials Laboratory, 1994.

### Drawing reprinted from Single Sheet Agricultural Mechanics Plans. University of Missouri-Columbia: Instructional Materials Laboratory, 1994.

### Drawing reprinted from Single Sheet Agricultural Mechanics Plans. University of Missouri-Columbia: Instructional Materials Laboratory, 1994.

### Activity 18									          SG 18.1

### Electronic Bill of Materials Scoring Guide

Name _________________________

| Exemplary 2 pts. | Acceptable 1 pt. | Needs Work 0 pts. | Pts. |
| --- | --- | --- | --- |
| ## Overall Organization |  |  |  |
| Meets all of the following criteria: 1. Headers provide appropriate labeling 2. Columns are in logical order 3. Logical flow of calculations | Meets two of the criteria | Meets one or none of the criteria |  |
| ## Text Mechanics |  |  |  |
| No errors in punctuation or spelling | A few minor errors that are not distracting | Numerous or distracting errors |  |
| ## Formulas |  |  |  |
| All formulas are entered correctly and calculate the data when entered | One formula is entered incorrectly | Two or more formulas are entered incorrectly |  |
| ## Content Accuracy |  |  |  |
| ## Meets all of the following criteria: 1. All materials, dimensions, prices, etc., are accurate 2. All lumber/metal is listed in most cost-efficient size 3. Total cost is accurate | ## One or two minor errors | ## Numerous errors |  |
| ## Content Completeness |  |  |  |
| All necessary information is entered | *** | Information is missing |  |
| ### Total points out of 10 _____ |  |  |  |

***No middle-ground criteria (either exemplary or needs work)