Agricultural Construction

Curriculum Guide:  Agricultural Construction Volume III

Unit:  III. Arc Welding (GTAW/TIG)

Unit Objective:

Students will apply principles of gas tungsten arc welding by performing common welds with a gas tungsten arc welding outfit, identifying welding equipment, and answering questions about related equipment and procedures.

Show-Me Standards: 1.10, CA3

References:

Agricultural Construction Volume I. University of Missouri-Columbia, Instructional Materials Laboratory, 1989.

Agricultural Construction Volume III. University of Missouri-Columbia, Instructional Materials Laboratory, 2002.

American Welding Society. Accessed November 18, 2003, from http://www.aws.org/.

ESAB Knowledge Centre. ESAB. Accessed November 25, 2003, from http://www.esab.com/.

Hobart Institute of Welding Technology. Accessed November 17, 2003, from http://www.welding.org/.

Lincoln Electric. Accessed November 18, 2003, from http://www.lincolnelectric.com/.

Machinery Safety: Welding. National Ag Safety Database. Accessed November 17, 2003, from http://www.cdc.gov/nasd/menu/topic/machinery_welding.html.

Miller Electric. Accessed November 18, 2003, from http://www.millerwelds.com/.

Missouri CDE Handbook. Accessed November 14, 2003, from http://www.dese.mo.gov/divcareered/ag_cde_guidelines.htm.

Missouri FFA Agricultural Mechanics Career Development Event. Accessed November 19, 2003, from http://web.missouri.edu/~pavt0689/statecon.html.

Instructional Strategies/Activities:

•	Students will engage in study questions in lesson 1.

•	Students will complete JS 3.1, Prewelding and Postwelding Procedures for GTAW; JS 3.2, Welds in the Flat Position; JS 3.3, Welds in the Horizontal Position; JS 3.4, Welds in the Vertical Position; and JS 3.5, Welds in the Overhead Position.

•	Additional activities that relate to the unit objective can be found under the heading “Other Activities” in the following location: p. 69.

Performance-Based Assessment:

Students will perform a series of welds determined by the instructor, identify gas tungsten arc welding equipment, and answer questions about gas tungsten arc welding equipment and procedures. This activity is modeled on the arc welding portion of the Agricultural Mechanics Career Development Event.

Assessment will be based on the ability to safely and correctly perform the assigned welding procedures and on the accuracy of responses to the identification and written assessment portions of the activity.

Agricultural Construction Volume III

Unit III—Arc Welding (GTAW/TIG)

Instructor Guide

The instructor should explain the performance-based assessment activity format at the beginning of the unit. Students will work toward completing the competencies necessary to perform the activity as they progress through the unit material. The assessment activity will be due at the completion of the unit.

1.	Explain the performance-based assessment activity format at the beginning of the unit: At the completion of the unit, students will perform a series of welds, identify welding-related equipment, and answer questions about welding equipment and procedures. Welds will be determined by the instructor and announced at the time of the performance-based assessment activity.

2.	Use or adapt the activity sheets found in the unit to assess student competency at welding with the gas tungsten arc welding outfit. Review or supplement these activities as needed, based on student mastery of the procedures and equipment the students will be using. NOTE: Students should only complete this performance-based activity if they have mastered all the relevant competencies and have the instructor’s permission to perform the activity.

3.	Assign the performance-based assessment activity. The student handout can be used as an outline for the activity or adapted as desired.

a.	Information and directions for the student handout as it is currently written are listed at the end of this instructor guide.

b.	Section II requires some advance setup by the instructor.

4.	This activity is modeled on the arc welding portion of the Agricultural Mechanics Career Development Event.

a.	Refer to the Missouri CDE Handbook for guidelines regarding Career Development Events. The Missouri CDE Handbook is available from the Missouri Department of Elementary and Secondary Education at http://www.dese.mo.gov/divcareered/ag_cde_guidelines.htm.

b.	Previous years’ agricultural mechanics events can be found at http://web.missouri.edu/~pavt0689/statecon.html, accessed July 7, 2003.

5.	Have students turn in their weld and completed handouts.

6.	The final assessment score will be based on the ability to safely and correctly perform the assigned welding procedures and on the accuracy of responses to the identification and written assessment portions of the activity. All welds must pass destructive testing.

7.	ADDITIONAL ACTIVITY: Create a display board using the students’ best welds. Possible display board themes include the following: each student’s best weld, the best example of each type of weld performed by the class, and the best weld of the week.

Section I: Welding

1.	Have students perform a series of welds that they have mastered as part of the instructional activities for this unit.

Section II: Identification

1.	Select ten parts of the gas tungsten arc welder or items of related equipment that have been discussed in class.

2.	Label the parts or items with tags A through J.

3.	Have students identify the parts on their handouts.

Section III: Written Assessment

1.	Have students answer questions about gas tungsten arc welding procedures, equipment, or safety. Multiple-choice and short-answer questions are suggested.

2.	The answers to the questions on the student handout are listed below.

Answers to Written Assessment:

1.	d

2.	c

3.	c

4.	b

5.	Students should list six of the following:

a.	Never drag cables or hoses or pull them to force them over an obstruction.

b.	Run hoses and cables so that they will not be damaged or cause a tripping hazard.

c.	Use only clean rags to clean cables and hoses.

d.	Keep cables and hoses free of kinks at all times.

e.	Do not drape welding cables or hoses over any type of gas cylinder or over the flowmeter or regulator.

f.	Never strike an arc on a gas cylinder.

g.	Cylinders must be fastened to a wall, post, or approved cylinder truck so that they stay upright at all times.

h.	Valve protection caps should be in place when cylinders are not in use.

i.	Crack cylinders before attaching the regulator to clean any debris out of the cylinder valve outlet.

Agricultural Construction Volume III

Unit III—Arc Welding (GTAW/TIG)

Student Handout

Section I: Welding

Directions:

1.	The instructor will give you a series of welds to perform.

2.	Perform the assigned welds.

	Wear appropriate safety equipment at all times.

	Follow all assigned safety procedures. You can lose points for not following safety precautions and other assigned procedures.

	Inspect the equipment, materials, and work area to ensure safe and correct operation.

	Perform the welds using the assigned procedure.

	Inspect your work.

	Follow shutdown and cleanup procedures and return all equipment and materials to their assigned places.

	Turn in your work to the instructor.

3.	Complete sections II and III of the activity and turn your completed handout in to the instructor.

4.	Your final assessment score will be based on your ability to safely and correctly perform the assigned welding procedures and on the accuracy of your responses to the identification and written assessment portions of the activity. All welds must pass destructive testing.

Section II: Identification

Directions:

Go to the identification station. Write the names of the tagged parts or items in the spaces below. Be sure to write each name next to its correct tag letter.

| A. | F. |
| --- | --- |
| B. | G. |
| C. | H. |
| D. | I. |
| E. | J. |

Section III: Written Assessment

Circle the letter that corresponds to the correct answer.

1.	Which of the following is not an advantage of gas tungsten arc welding (GTAW)?

a.	Welds are generally clean.

b.	Welding is easily done in all positions.

c.	The arc and weld pool are clearly visible.

d.	GTAW is cheaper and faster than shielded metal arc welding.

2.	________ is an inert shielding gas used in GTAW.

a.	Oxygen

b.	Hydrogen

c.	Argon

d.	Nitrogen

3.	Which of the following is a step in GTAW setup?

a.	Turn the power on and attach the torch hoses.

b.	Disconnect the ground clamp.

c.	Set the current range.

d.	Close the gas cylinder valve.

4.	Which of the following is a step in GTAW shutdown?

a.	Adjust the electrode extension.

b.	Bleed the gas line.

c.	Crack the cylinder valve.

d.	Select the collet body, collet, and nozzle.

Complete the following short-answer question.

5.	List six safety and maintenance procedures for working with welding cables, hoses, and cylinders. (Each answer is worth 1 point for a maximum value of 6 points.)

a.

b.

c.

d.

e.

f.

Agricultural Construction

Agricultural Construction Volume III

Unit III—Arc Welding (GTAW/TIG)

Scoring Guide	Name

| Assessment Area Section I | Criteria | 0 Points | 1 Point | 2 Points | 3 Points | 4 Points | Weight | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  Page 11  Positioning | Metal was positioned properly | Failed | Poor | Fair | Good | Excellent | X 4 |  |
| Equipment Setup | Equipment was properly adjusted | Failed | Poor | Fair | Good | Excellent | X 4 |  |
| Distortion | Welds show no signs of distortion | Failed | Poor | Fair | Good | Excellent | X 3 |  |
| Appearance | Weld beads are uniform | Failed | Poor | Fair | Good | Excellent | X 5 |  |
| Strength | Welds are strong and sound | Failed | Poor | Fair | Good | Excellent | X 4 |  |
| Safety and Work Habits | Student followed all safety precautions | Passed |  |  |  | Failed | X (-20) | Negative Points * |
|  | Student followed all assigned procedures | Excellent | Good | Fair | Poor | Failed | X (-8) | Negative Points * |
| TOTAL |  |  |  |  |  |  |  | /80 pts. |

| Assessment Area |  |  |  |  |  |  |  |  | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Section II: Identification |  |  |  |  |  |  |  |  |  |
| Section III: Written Assessment |  |  |  |  |  |  |  |  |  |
| TOTAL |  |  |  |  |  |  |  |  | /20 pts. |

 Page 12 

Final Assessment Total ________/100 pts.

* Overall combined score cannot be lower than 0.

Comments: