Unit II – Arc Welding							      AS 3 (Student)

Lesson 5: Welding Out of Position

# Making an Uphill Fillet Weld

Objective: Students will make an uphill fillet weld using a shielded metal arc welder.

Directions: Students will use an arc welder to make an uphill fillet weld in a tee joint.

Materials and Equipment:

SMAW machine and accessories

Chipping hammer

Wire brush

Helmet*

Safety glasses or goggles

Leather gloves and any other protective clothing recommended

by instructor

SMAW electrode(s), selected by instructor

Mild steel plates, selected by instructor

* Everyone participating in or observing the demonstration should wear appropriate protective eyewear.

Procedure:

Figure 1

1.	Inspect equipment, materials, and work area to ensure safe and correct operation.

2.	Wear appropriate face and eye protection and protective clothing.

3.	Set up and turn on the machine following assigned procedure.

4.	Cover up and remind those in the area to do so as well.

5.	Tack weld the pieces together at a 90-degree angle, leaving approximately a 1/16-in. gap between the pieces. Refer to Figure 1.

6.	Clean slag from the tack welds.

Figure 2

7.	Secure the pieces in the vertical position.

8.	Strike an arc and weld the joint.

a.	Hold the electrode approximately 10 to 15 degrees below a right 	angle, 	so that it is pointed up at the work, and about 45 degrees 	from each side. Refer to Figures 2 and 3.

b.	Generally, the arc length used for vertical welds is shorter than 	that used for flat welds.

Figure 3

c.	The electrode can be moved ahead, out, and back to the weld 	pool to help control the temperature of the pool. The arc is not 	broken in this movement. Refer to Figure 4.

9.	Remove the slag from the weld.

10.	Run additional passes if needed to complete the weld, cleaning the weld between each pass. A weaving pattern can be used to distribute heat if needed.

11.	Clean the final pass and inspect the weld.

Figure 4

12.	Remove the electrode from the holder and observe safety, shutdown,

and cleanup procedures.

13.	Turn in work to be graded by the instructor.