Unit IV - Tool Sharpening and Reconditioning	           Name_________________________

Lesson 1: Sharpening and Reconditioning	           Date__________________________

a Twist Drill, Lawn Mower Blade, and Chain Saw Chain

# Assessment

# Circle the letter that corresponds to the correct answer.

1.	Before changing a wheel on a grinder, the operator should:

a.	unplug the grinder.

b.	dress the new wheel.

c.	let the wheel run for a minute.

d.	put the guards in operating position.

2.	How should an object being ground be moved on the grinding wheel?

a.	Side to side on the side of the wheel

b.	Up and down on the side of the wheel

c.	Side to side on the face of the wheel

d.	Up and down on the face of the wheel

3.	As the grit number of grinding wheels decreases, the abrasives become:

a.	harder.

b.	softer.

c.	coarser.

d.	finer.

4.	Which grade of wheel is commonly selected for tool sharpening?

a.	Hard

b.	Medium hard

c.	Medium soft

d.	Soft

5.	At what degree angle should the cutting lip of a twist drill be to the center of the drill?

a.	25

b.	32

c.	47

d.	59

6.	What tool is used to measure the tip of a twist drill?

a.	Tool gauge

b.	File guide

c.	Depth gauge

d.	Blade balancer

7.	What tool maintains the file at proper depth and angle when sharpening a chain saw chain?

a.	Blade balancer

b.	Depth gauge

c.	File guide

d.	Tool gauge

8.	What is the typical angle on a lawn mower blade?

a.	35

b.	45

c.	55

d.	65

9.	A link in a chain saw chain is called a:

a.	Toe

b.	Heel

c.	Gullet

d.	Cutter

10.	Which two tools are typically used to sharpen a chain saw chain?

a.	Round file and file guide

b.	Round file and grinder

c.	Depth gauge and file guide

d.	Depth gauge and grinder

# Complete the following short-answer questions.

11.	What is the purpose of dressing a grinding wheel?

12.	List three specifications to consider when choosing a grinding wheel and describe what each specification means.

a.

b.

c.

13.	What are the three parts of a tip on a twist drill?

a.

b.

c.

14.	What protective clothing is recommended for removing and installing a lawn mower blade and why?

15.	Why is it important to balance a lawn mower blade?

16.	List three signs that a chain saw chain needs sharpening.

a.

b.

c.

17.	What can happen if the tension of a chain saw chain is too loose?