Unit V - Cold Metal Work				           Name_________________________

Lesson 1: Working With Cold Metal		           Date__________________________

# Assessment

# Circle the letter that corresponds to the correct answer.

1.	Which tool is designed to scribe arcs and circles on metal?

a.	Center punch

b.	Dividers

c.	Scratch awl

d.	Soapstone

2.	Which tool is designed to mark the center of a hole for starting a twist drill?

a.	Soapstone

b.	Scratch awl

c.	Dividers

d.	Center punch

3.	When cutting with a hacksaw, how many teeth should be on the metal at all times?

a.	Two

b.	Three

c.	Four

d.	Five

4.	Which type of cold chisel is best for shearing metal?

a.	Cape

b.	Diamond

c.	Flat

d.	Round nose

5.	For best results when cutting with snips, the operator should:

a.	close the blades completely on a cut.

b.	open the blades wide to improve leverage.

c.	keep the metal at the front part of the blades.

d.	prevent the metal from lifting up during the cut.

6.	Which tool would probably be best for cutting structural steel?

a.	Cold chisel

b.	File

c.	Power shears

d.	Snips

7.	Which cutting design of a file has individual teeth that are raised and sharp?

a.	Curve

b.	Double

c.	Rasp

d.	Single

8.	Which tools or materials are used to round cold metal?

a.	Ball-peen hammer and pipe

b.	Ball-peen hammer and angle iron

c.	Vise and adjustable wrench

d.	Vise and angle iron

9.	When fastening metal with a 3/16-in. diameter rivet, how far should the rivet extend above the metal?

a.	1/16 in.

b.	1/8 in.

c.	3/16 in.

d.	1/4 in.

10.	A cutting tool designed to cut external threads on round metal is called a:

a.	tap drill.

b.	tap.

c.	die stock.

d.	die.

11.	Which type of thread system is typically used for fastening metal that will be under high-stress situations?

a.	NC

b.	NF

c.	ISO coarse

d.	NPT

# Complete the following short-answer questions.

12.	Define the following terms as related to a saw blade.

a.	Pitch -

b.	Set -

13.	How should a hacksaw blade be installed?

14.	What is the correct way to hold a file?

15.	What part of a file does the handle go on? Why is it important to use a handle when

filing?

16.	Why is it important to not overtighten a screw?

17.	What is the function of a rivet set?

18.	What is a taper tap used for?

19.	What is the correct tap drill size for a 3/4 - 16NF tap? Show your work.

20.	List two reasons that burrs should be removed from metalworking projects.

a.

b.