Unit V - Cold Metal Work				     			      AS 2 (Instructor)

Lesson 1: Working With Cold Metal

# Fastening Metal With Rivets and Pop Rivets

# Objective: Students will observe how to fasten metal using different types of rivets.

Directions: Use rivets and pop rivets to fasten metal.

# Materials and Equipment:

| Safety goggles or glasses* Gloves Ball peen hammer Portable or stationary drill and appropriate twist drill Center punch Rivet set | Rivets Riveting pliers Pop rivets Bench vise Metal stock, selected by the instructor Anvil or other appropriate work surface |
| --- | --- |

* Everyone participating in or observing the demonstration should wear appropriate

protective eyewear and any other protective clothing as needed.

NOTE: The following demonstrations are intended to illustrate fastening techniques

covered during the classroom discussion. The demonstrations below are of techniques

presented in the lesson. If additional or different techniques were covered in class, they

should be added to or substituted for those below.

# Procedure:

1.	Wear appropriate safety equipment.

2.	Demonstrate the correct procedure for fastening metal using rivets.

a.	Use the center punch to lay out the center of the hole on two pieces of metal.

b.	Secure one piece of stock and drill a hole in it the same diameter as the rivet to be 	used. Repeat with the second piece.

c.	Choose a rivet of the correct length. A general rule for selecting a rivet is that it 	should be as long as the thickness of both pieces of metal together plus the 	diameter of the rivet. For example, a 1/8-in. diameter rivet should protrude 	approximately 1/8 in. above the two pieces of stock.

d.	Put the pieces on the rivet and set the assembled pieces, with the head of the 	rivet down, on an anvil or other appropriate base. For round-head rivets, a 	support with a recess that fits the rivet head can be used.

e.	Use a hammer and a rivet set to draw the pieces together.

f.	Strike the rivet squarely in the center, using the face of the hammer, to expand 	the rivet and tighten the pieces.

g.	Use the peen of the hammer or a rivet set to form a rounded rivet head on the 	exposed end. Do not flatten the rivet head; this will weaken the rivet.

3.	Demonstrate the correct procedure for fastening metal using pop rivets.

a.	Mark both pieces of metal using the punch.

b.	Drill a hole in both pieces that is the same size as the rivet to be used.

c.	Choose rivets that are compatible with the metal being fastened.

d.	Make sure the pieces are held firmly together. Put the pin of the rivet in the pop 	riveter and put the rivet in the hole.

e.	Draw the rivet up by squeezing the handles until the pin breaks. This may 	require several strokes.

4.	Demonstrate the correct cleanup procedure.