Unit IV - Tool Sharpening and Reconditioning			     	          AS 3 (Student)

Lesson 1: Sharpening and Reconditioning a Twist Drill, Lawn Mower Blade,

and Chain Saw Chain

# Sharpening a Lawn Mower Blade

Objective: Students will sharpen a lawn mower blade.

Directions: Students will use a file or portable grinder to sharpen a lawn mower blade.

# Materials and Equipment:

Portable grinder with appropriate grinding wheel or

Appropriate flat file(s)

Safety goggles, face shield, or other approved face and eye protection*

Gloves, for removing and installing the blade and any filing procedures

Any other protective clothing recommended by instructor

Tool gauge with the correct angle for the sharpened blade

Socket wrench and socket

Bench vise

Blade balancer or a thin edge for checking the balance of the blade

Lawn mower with a blade that needs sharpening

* Everyone participating in or observing the demonstration should wear appropriate

protective eyewear and any other protective clothing as needed.

Procedure:

1.	Wear appropriate safety equipment.

2.	Use the correct procedure for preparing to sharpen the lawn mower blade.

a.	Consult the owner’s manual for the correct angle for sharpening the blade or any 	other relevant information. A typical angle for a properly sharpened lawn 	mower blade is 45 degrees.

b.	Inspect equipment, materials, and work area to ensure safe and correct 	operation.

3.	Disconnect the spark plug wire and secure it away from the spark plug. Disconnect an electric mower from its power source.

4.	Follow the proper procedure for removing the mower blade, including the following steps and any others as directed by the instructor.

a.	Tip the mower on its side, with the crankcase down.

b.	Secure the mower so that it will not fall over while it is being worked on.

c.	Use the socket wrench and socket to remove the blade.

5.	Follow the correct procedure for sharpening the mower blade.

a.	Secure the blade in the vise.

b.	Use the grinder or file to remove any nicks in the blade.

1. When using a file, apply pressure on the forward stroke and lift the file for the 		    return stroke.

2. When using a grinder, be careful not to overheat the metal.

3. When using a file or grinder, be careful to remove metal evenly from both 	    	    sides of the blade to avoid throwing it out of balance.

c.	Use the sharpening tool to restore the flat side of each end of the blade.

d.	Restore the cutting edge of each end of the blade.

e.	Use the tool gauge to check the angle of each end of the blade.

f.	Use the blade balancer or a thin edge to check the balance of the blade.

g.	If needed, reclamp the blade and carefully file or grind the heavy side of the 	blade until it is in balance.

6.	When the blade is balanced and the cutting edges have the correct angle, reinstall the blade.

7.	Set the mower back down in the cutting position and reattach the spark plug wire.

8.	Perform assigned cleanup procedures. Materials and equipment should be returned to their proper places.

9.	Turn in work to be graded by the instructor.