Unit IV - Tool Sharpening and Reconditioning			     	      AS 1 (Instructor)

Lesson 1: Sharpening and Reconditioning a Twist Drill, Lawn Mower Blade,

and Chain Saw Chain

# Dressing a Grinding Wheel

Objective: Students will observe how to dress a grinding wheel.

Directions: Use a mechanical wheel dresser to dress a grinding wheel.

Materials and Equipment:

| Bench grinder Grinding wheel that needs reconditioning Mechanical wheel dresser or other appropriate dressing tool | Face shield or other approved face and eye protection* Dust mask or respirator Leather apron or any other protective clothing recommended by instructor |
| --- | --- |

* Everyone participating in or observing the demonstration should wear appropriate

protective eyewear, a respirator, and any other protective clothing as needed.

# Procedure:

1.	Wear appropriate safety equipment.

2.	Demonstrate the correct setup procedure for dressing a grinding wheel.

a.	Inspect equipment, materials, and work area to ensure safe and correct 	operation.

b.	With the power disconnected, adjust the tool rest and make sure all guards are in 	place.

c.	When all adjustments are made, plug in the grinder.

d.	Stand to the side of the grinder and turn it on. Let the grinder run before using.

3.	Demonstrate the correct procedure for dressing the grinding wheel.

CAUTION: Be prepared for particles of the abrasive grit to fly off the grinding wheel. The wheel is trued and dressed by removing old abrasive and waste material clogging the wheel surface.

a.	Position the wheel dresser on the grinder.

b.	Raise the dresser handle to push the dresser wheels firmly against the grinding 	wheel.

c.	Move the dresser back and forth across the grinding wheel, maintaining even 	pressure on the dresser.

d.	If there is heavy sparking, increase the pressure on the dresser. Excessive 	sparking indicates that the dresser is being ground away.

4.	Turn off the grinder, disconnect it from the power source, and examine the wheel.

a.	It should be perfectly round and the face should be square with the sides.

b.	The grit should be sharp and free of waste particles.

c.	There should be no grooves in the surface.

5.	Repeat dressing procedure if additional dressing is needed.

6.	Demonstrate correct shutdown procedure.

7.	Readjust the tool rest so that it is 1/16 in. from the wheel.

8.	Assign the student version of AS 1 to be performed by the students.