Unit VI - Material Selection, Plan Reading, 		           Name_________________________

and Interpretation

Lesson 3: Selecting Building Materials			           Date__________________________

# Assessment

# Circle the letter that corresponds to the correct answer.

1.	Which of the following is a hardwood?

a.	Ash

b.	Cedar

c.	Pine

d.	Redwood

2.	Softwood that has a nominal thickness ranging from 2 in. to 5 in. is called:

a.	Boards

b.	Dimensional lumber

c.	Stock

d.	Timbers

3.	Lumber that has been surfaced on all sides and one edge is designated:

a.	G1S

b.	G2S

c.	S2S1E

d.	S4S1E

4.	Which grade is the lowest quality for softwood used for finishing?

a.	A

b.	B

c.	C

d.	D

5.	Which hardwood grade is the best quality?

a.	Selects

b.	Firsts and Seconds

c.	Number 1 Common

d.	Number 2 Common

6.	In a board measurement of 2" x 6" x 16', what does the 6" measurement refer to?

a.	Height

b.	Length

c.	Thickness

d.	Width

7.	Which softwood plywood grade is the best quality?

a.	N

b.	C

c.	B

d.	A

8.	Plywood is measured and sold by the:

a.	board foot

b.	linear foot

c.	running foot

d.	square foot

Match the description in column B to the correct structural metal in column A.

| A | B |
| --- | --- |
| _____ 9. Round | a. Shaped like a C |
| _____ 10. Angle iron | b. Two legs that form a 90-degree angle |
| _____ 11. Flats and strips | c. Shaped like a tube |
| _____ 12. Channel iron | d. Bar of solid metal |
| _____ 13. Steel pipe | e. Flat pieces of metal |

Complete the following short-answer questions.

14.	Define the following terms relating to lumber.

a.	Softwood -

b.	Hardwood -

c.	Nominal size -

d.	Actual size -

15.	List three ways that softwood is classified.

a.

b.

c.

16.	Calculate the board feet of the following lumber:

a. 	Seven boards measuring 1/2" x 4" x 12' = _________ bd ft

b.	Two boards measuring 2" x 6" x 8' = __________ bd ft

17.	Calculate the total cost of the following lumber:

a.	Five boards measuring 1" x 8" x 18' and priced at $0.40 per board 
		foot = ______total cost

b.	Three boards measuring 2" x 6" x 8' and priced at $0.32 per board 
		foot = ______total cost