Unit II – Arc Welding				           Name_________________________

Lesson 3: Selecting and Maintaining		           Date__________________________

Electrodes and Safety Lenses

Assessment

# Complete the following short-answer questions.

1.	List five factors to consider when choosing an electrode.

a.

b.

c.

d.

e.

2.	Answer the following questions regarding an E6011 electrode.

a.	What information does the “E” provide?

b.	What is the minimum tensile strength of the weld metal? Include the unit of measurement for tensile strength.

c.	In what position or positions can the electrode be used?

3.	List three reasons why electrodes must be kept dry.

a.

b.

c.

4.	List three procedures for caring for filter lenses or the welding helmet.

a.

b.

c.

# Circle the letter that corresponds to the correct answer.

5.	The last digit in an AWS classification number indicates:

a.	electrode diameter.

b.	tensile strength.

c.	welding current.

d.	welding position.

6.	In which welding positions can an E6020 electrode be used?

a.	All positions

b.	Flat and horizontal

c.	Overhead and vertical down

d.	Designator no longer used