Unit III – Oxyacetylene Welding			           Name_________________________

Lesson 2: Welding With Oxyacetylene		           Date__________________________

Assessment

# Circle the letter that corresponds to the correct answer.

1.	Differences between an oxyacetylene cutting outfit and a welding outfit can be found in the:

a.	hoses and acetylene control valve.

b.	hoses and oxygen control valve.

c.	torch and acetylene regulator.

d.	torch and oxygen regulator.

2.	If the torch has a tip that is the incorrect size for the work, what should the welder do?

a.	Adjust the flow of oxygen.

b.	Adjust the flow of acetylene.

c.	Replace the tip with the correct size.

d.	Recondition the tip on a grinding wheel.

3.	In oxyacetylene welding, what is a usual torch angle and distance to hold the flame from the work?

a.	45 degrees and 1/8 to 1/4 in.

b.	45 degrees and 3/8 to 1/2 in.

c.	60 degrees and 1/8 to 1/4 in.

d.	60 degrees and 3/8 to 1/2 in.

4.	A small diameter welding rod can be used on thick metal to:

a.	produce better penetration.

b.	avoid burnthrough.

c.	add more filler material than a larger rod.

d.	remove excess heat from the weld pool.

5.	A common welding problem in which the oxyacetylene flame goes out with a snapping sound is called:

a.	backfire.

b.	carbonization.

c.	flashback.

d.	oxidation.

6.	A correct statement about a carburizing flame is that it:

a.	has two distinct parts.

b.	has a high temperature.

c.	contains too much oxygen.

d.	may add carbon to the weld.

7.	A correct statement about an oxidizing flame is that it:

a.	is low temperature.

b.	makes a lot of noise.

c.	has a longer inner cone.

d.	contains too much fuel.

# Complete the following short-answer questions.

8.	Answer the following questions regarding an RG-60 welding rod.

a.	What does the “R” indicate?

b.	What does the “G” indicate?

c.	What does the “60” indicate?

9.	List two reasons why wire or rods that are not classified for oxyacetylene welding should not be used.

a.

b.

10.	Define the following terms.

a.	Forehand welding -

b.	Backhand welding -

11.	What should be done if a backfire occurs?

12. What should be done if a flashback occurs?