Unit VII - Painting and Finishing							   AS 1 (Student)

Lesson 1: Spray Painting and Finishing

# Spray Painting

Objective: Students will finish a project using spray equipment.

Directions: Students will use spray equipment to apply primer and finish.

# Materials and Equipment:

| Spray equipment and accessories Safety goggles* Respirator Rubber gloves and any other protective clothing recommended by instructor Wooden paint stirring paddle Unpainted shop project Appropriate paint Compatible primer | Thinner Solvent cleaner Paint strainers Masking tape Clean cloths Clean containers for mixing and storing paint Paper, cardboard, or scrap material for testing spray pattern Mineral spirits to clean spray gun, if water-based finish is used |
| --- | --- |

* Everyone participating in or observing the demonstration should wear appropriate

protective eyewear, a respirator, and any other protective clothing as needed.

# Procedure:

1.	Wear appropriate safety equipment.

2.	Prepare the surface to be finished following assigned steps. Procedures include, but aren’t limited to, the following.

a.	Clean the surface.

b.	Repair surface imperfections.

c.	Use masking tape to cover areas that should not be painted.

3.	Prepare the primer coat following manufacturer instructions.

4.	Set up the spray equipment following assigned procedure.

a.	Set air pressure.

b.	Adjust the spray gun.

c.	Test the spray pattern using the paper or cardboard.

d.	Make any adjustments to the spray pattern, if needed.

5.	Apply primer with the spray equipment using techniques discussed in the lesson and following an appropriate spraying sequence. Refer to Figures 1 and 2.

a.	Hold the gun the correct distance from the work. For 	air spray equipment, this is approximately 8 in.

b.	Paint corners and edges first.

c.	Keep the gun at a 90-degree angle to the work.

d.	Move the gun parallel to the work through the whole 	pass.

e.	Begin and end spraying with the gun in motion.

Figure 1

f.	Overlap passes by 50%.

6.	Clean the spray gun.

a.	Empty the paint cup.

b.	Use an appropriate solvent for cleaning.

c.	Do not use wires or other metal objects to clean 	orifices on the paint gun.

d.	Dry all parts and reassemble the spray gun.

e.	If the sprayer has been used to spray water-based 	material, run mineral spirits through the gun to 	remove any moisture and keep the gun from rusting.

f.	Dispose of cleaning cloths properly.

g.	Dispose of or store any unused primer properly.

Figure 2

7.	When the primer coat is dry, prepare the finish.

a.	Mix the finish.

b.	Thin the finish, if needed.

c.	Strain the finish.

8.	Finish the project using an appropriate spraying sequence.

9.	Clean the spray gun.

10.	Dispose of cleaning cloths properly.

11.	Dispose of or store any unused finish properly.

12.	Return materials and equipment to their assigned places.

13.	Turn in work to be graded by the instructor.