Unit II – Arc Welding				           Name_________________________

Lesson 1: Safety and Maintenance 		           Date__________________________

Procedures for Arc Welding

Assessment

Read the following statements from Welders A and B. Indicate which is correct by selecting from the four listed choices.

1.	Welder A says that putting out an electrical fire with water does not cause a hazard for electrical shock.

Welder B says that changing the current while the welder is under load does not cause a hazard for electrical shock.

a.	Welder A is correct.

b.	Welder B is correct.

c.	Both are correct.

d.	Neither is correct.

2.	Welder A says standing on steel while welding is not an electrical hazard.

Welder B says that the electrode should be removed from the holder when welding is completed to avoid a shock hazard.

a.	Welder A is correct.

b.	Welder B is correct.

c.	Both are correct.

d.	Neither is correct.

3.	Welder A says that the walls of the welding booth should be made of wood to avoid a fire hazard.

Welder B says that the floor should be free of cracks to avoid a fire hazard.

a.	Welder A is correct.

b.	Welder B is correct.

c.	Both are correct.

d.	Neither is correct.

4.	Welder A says that hot metal pieces should be handled with tongs rather than gloved hands.

Welder B says that rubber gloves should be worn while welding to protect the hands from burns.

a.	Welder A is correct.

b.	Welder B is correct.

c.	Both are correct.

d.	Neither is correct.

5.	Welder A says that a light-colored short-sleeved shirt should be worn for welding.

Welder B says that polyester pants with cuffs should be worn for welding.

a.	Welder A is correct.

b.	Welder B is correct.

c.	Both are correct.

d.	Neither is correct.

6.	Welder A says that dark, tightly woven clothing should be worn to protect the skin from harmful arc rays.

Welder B says that safety lenses with filter lens of no. 10 or higher should be worn for chipping slag from a weld.

a.	Welder A is correct.

b.	Welder B is correct.

c.	Both are correct.

d.	Neither is correct.

7.	Welder A says that good ventilation is essential for safe welding.

Welder B says that a respirator is sometimes required to avoid exposure to harmful fumes from some metals.

a.	Welder A is correct.

b.	Welder B is correct.

c.	Both are correct.

d.	Neither is correct.

8.	Welder A says that grease should be applied to the cables to prevent them from drying out.

Welder B says that the welder should not be turned on while the electrode holder is in contact with the work.

a.	Welder A is correct.

b.	Welder B is correct.

c.	Both are correct.

d.	Neither is correct.

9.	Welder A says that people in the work area do not need to wear protective eyewear if the welding booth has a curtain.

Welder B says that it is necessary to warn others in the area that welding is about to begin so they can protect themselves.

a.	Welder A is correct.

b.	Welder B is correct.

c.	Both are correct.

d.	Neither is correct.

# Complete the following short-answer question.

10.	 List ways to avoid the safety hazards listed below.

a.	Electric shock

1)

2)

3)

b.	Burns and fire

1)

2)

3)

c.	Burns from arc rays

1)

2)

d.	Breathing hazards

1)

2)

3)