UNIT III - BASIC LABORATORY SKILLS

Lesson 3:	Biotechnology Laboratory Safety

Competency/Objective:	Explain why safety practices should be followed in the laboratory.

Study Questions

- What are some of the common biotechnology laboratory safety concerns?

- How is a spill cleaned up?

- What are the methods for disposing of materials used in biotechnology?

- What procedures should be followed in case of fire?

- What personal protective equipment should be worn in the laboratory?

- What should be done if an injury occurs?

- What ventilation is needed in a laboratory?

References

1.	Biotechnology:  Applications in Agriculture (Student Reference).  University of Missouri-Columbia: Instructional Materials Laboratory, 1998, Unit III.

2.	Activity Sheet

a)	AS 3.1:	Using a Material Safety Data Sheet (MSDS)

UNIT III - BASIC LABORATORY SKILLS

Lesson 3:	Biotechnology Laboratory Safety

TEACHING PROCEDURES

- Review

Lesson 2 discussed some of the equipment used in biotechnology laboratories as well as aseptic techniques to reduce contamination of laboratory experiments.  This lesson will examine the topic of laboratory safety.  Laboratory safety is always important, especially in biotechnology laboratories.  In the late 1970s and early 1980s, laboratories doing genetic manipulation were contained in a nearly sealed environment because of the fear of releasing pathogens, as well as other unknown risks.  Today researchers have a much better understanding of the dangers of such research and take necessary precautions when needed.  All laboratory research poses some risks that can be addressed by following a detailed safety plan.  Laboratory safety is a top priority in every biotechnology laboratory.

- Motivation

Before students arrive for class, create a “chemical spill” on a countertop in the laboratory that students will notice as they arrive.  The chemical spill can simply be water mixed with a few tablespoons of baking soda and a teaspoon of lemon juice.  Do not use an actual laboratory chemical or something that will stain the countertop.  When students arrive, explain that the spill must be cleaned up, but the correct process must first be determined.  Ask them how they would go about cleaning up the spill.  Have students speculate as to what types of spills might occur in a biotechnology laboratory.

- Assignment

- Supervised Study

- Discussion

  - Ask students to list the safety concerns that they would have if they were working in a biotechnology laboratory.

What are some of the common biotechnology laboratory safety concerns?

    - Microorganisms - Pathogenic microbes are dangerous because they can cause disease, and  even nonpathogenic microorganisms can be harmful in certain cases.

    - Chemicals

    - Radioactivity - Researchers must be trained to handle and dispose of radioactive materials safely.

    - Electrical hazards - Electrophoresis equipment can cause electrical shock.

    - Physical hazards - Centrifuges can injure fingers, and the ultraviolet light from transilluminators can damage retinas and skin.

  - Ask students to list the types of substances that might be spilled in a biotechnology laboratory (acids, bases, stains, microbe cultures, buffer solutions, etc.).  Discuss the clean up procedure and demonstrate the procedure by cleaning up the spill.

How is a spill cleaned up?

    - When the spilled chemical is known, the Material Safety Data Sheet (MSDS) outlines the proper clean up procedures.

    - When the substance is an unknown liquid, it should be absorbed using a special spill pillow;  the spill pillow should then be disposed of as hazardous waste.

    - If the spill is an unknown solid or powder, it can be gently swept into a glass container and disposed of as hazardous waste.

    - Next, the spill area should be cleaned with a disinfectant and an ethanol solution.

  - Discuss with students the different types of waste generated in a biotechnology laboratory and how wastes should be discarded.

What are the methods for disposing of materials used in biotechnology?

    - All cultures and equipment that come in contact with microbes should be autoclaved or disinfected with hospital-type disinfectants before being thrown away.

    - The MSDS should be followed for chemicals.

  - Ask students to if they know the classroom fire plan.  Discuss the procedures to be followed if a fire breaks out.

What procedures should be followed in case of fire?

    - The fire exit plan should be practiced during fire drills and followed when a fire occurs.

    - Everyone should know the location of the fire extinguisher, fire blanket, and fire alarm switch.

  - Ask students to describe the personal protective equipment (PPE) needed for handling pesticides.  Compare this list with the PPE needed for most biotechnology laboratories.

What personal protective equipment should be worn in the laboratory?

    - Safety glasses or goggles

    - Disposable latex gloves

    - Lab coat or apron

  - Ask students if they know what to do for various types of injuries.

What should be done if an injury occurs?

    - Simple first-aid procedures, like applying pressure to stop blood loss or flushing skin or eyes with water if they come in contact with chemicals, should be done immediately.

    - In a classroom lab, the instructor should also be notified without delay so that he or she can follow the school procedure for emergencies.

  - Ask students to list some of the airborne hazards found in biotechnology laboratories (gases, fine powders, microbes, etc.).

What ventilation is needed in a laboratory?

    - Fume hoods are used to remove bad odors or harmful vapors from the laboratory and to maintain a sterile environment for certain laboratory procedures.

    - Ventilated lockable chemical storage cabinets prevent the build up of gases that could cause an explosion or a fire.

- Other Activities

  - Show a video on laboratory safety, such as the video Beginning Chemistry Laboratory available from Carolina Biological Supply Company.

  - Use Glo-Germ to show the need for correct manipulation of bacteria and fungi.  Glo-Germ is a substance that is visible only under a black light.  Place Glo-Germ in a petri dish to represent a culture.  Have students perform tasks such as culture transfers, adding nutrients or changing the culture media, or tissue culture procedures.  The spread of Glo-Germ to other surfaces represents “contamination” of work areas.  If the Glo-Germ is not present on any surfaces, then the students have been successful.

- Conclusion

Safety in the biotechnology laboratory is critical.  Careful, safe working habits help produce successful research results.  If chemicals, cultures, or equipment are mishandled, the research is exposed to unnecessary hazards and may fail.  Researchers can work safely and effectively if proper precautions are taken in the laboratory.

- Answers to Activity Sheet

AS 3.1

  - Students may list any five of the following: chemical product and company identification; composition, information on ingredients; hazards identification; first aid measures; fire fighting measures; accidental release measures; handling and storage; exposure controls, personal protection; physical and chemical properties; stability and reactivity; toxicological information; ecological information; disposal considerations; transport information; regulatory information; and other information.

  - In a manner consistent with federal, state, and local regulations

  - Strong oxidizing agents; may form an explosive mixture with fluorine or potassium nitrate

  - It is not listed as a carcinogen.

  - Get medical aid.  Flush skin with plenty of soap and water for at least 15 minutes while removing contaminated clothing and shoes.

  - Odorless to a slightly acetic-like odor

  - Safety glasses with side shields, gloves, protective clothing to prevent skin exposure, and a NIOSH/MSHA-approved air purifying dust or mist respirator

  - It may cause respiratory and digestive tract irritation.  It may also cause eye and skin irritation.

- Answers to Evaluation

  - b

  - d

  - a

  - The fire exit plan should be practiced during fire drills and followed when a fire occurs.  Everyone should know the location of the fire extinguisher, fire blanket, and fire alarm switch.

  - Safety glasses or goggles, disposable latex gloves, and a lab coat or apron

  - Spill pillow

  - Flush the area with water and notify the instructor.

  - d

  - c

  - e

  - b

  - a

UNIT III - BASIC LABORATORY SKILLS				Name

Lesson 3:	Biotechnology Laboratory Safety				Date

EVALUATION

Circle the letter that corresponds to the best answer.

- What equipment is used in a laboratory for ventilation?

  - None

  - Fume hood

  - Fume cabinet

  - Open window

- An MSDS contains important information about:

  - Radioactive substances.

  - Specialty laboratory equipment.

  - Cultures of microorganisms.

  - Chemical substances.

- Before microbial cultures are disposed of they should be:

  - Autoclaved.

  - Sterilized.

  - Sealed in an airtight container.

  - Carefully scrubbed.

Complete the following short answer questions.

- What procedures should be followed in a laboratory to safeguard against fires?

- What personal protective equipment (PPE) should be worn in the laboratory?

- What should be used to clean up an unknown liquid that has spilled?

- What should be done if acid is spilled on a student’s hand in the laboratory?

Match the following hazards with an action in the second column that will help to protect the biotechnology researcher from the hazard.

- Physical hazards

- Electrical hazards

- Radioactivity

- Chemicals

- Microorganisms

a.	Aseptic techniques

b.	Following the MSDS

c.	Careful use of electrophoresis equipment

d.	Careful use of centrifuges and transilluminators

e.	Special training

UNIT III - BASIC LABORATORY SKILLS	AS 3.1

Lesson 3:	Biotechnology Laboratory Safety				Name

Using a Material Safety Data Sheet (MSDS)

Objective:	Identify important information from an MSDS.

Using the MSDS provided, answer the following questions.

- What are five of the major sections common to an MSDS sheet?

- How should the chemical be disposed of, according to the MSDS sheet?

- With what materials is the chemical incompatible?

- Is the chemical a known carcinogen?

- What first aid measures should be taken if the chemical comes in contact with the skin?

- Does the chemical have an odor?

- What personal protective equipment should be worn when using the chemical?

- What are the health effects of this chemical?