Unit II: Soil Management	Name

Lesson 2: Using Soil Survey Manuals	Date

Evaluation

Circle the letter that corresponds to the best answer.

1.	Where can soil survey manuals be obtained?

- a.	Local US post office

- b.	Soil and Water Conservation district

- c.	Farm Credit Service

- d.	US Soil Evaluation Center

2.	Soil maps for more detailed land use planning show the land using what type of pictures?

- a.	Aerial

- b.	Panoramic

- c.	Color

- d.	Topographic

Correctly match the appropriate drainage class with the description of that class.

a.	Well drained

b.	Excessively drained

c.	Somewhat poorly drained

d.	Very poorly drained

e.	Somewhat excessively drained

f.	Moderately well drained

g.	Poorly drained

____  3.	Very porous; freely permeable for great depths

____  4.	Wet for many months of the year; mottling prominent throughout the profile; artificial drainage required for crop growth

____   5.	Slower internal water movement; more mottling further up the profile; artificial drainage suggested for alfalfa crop

____  6.	Wet almost every month of the year; usually gray in color with mottling on the surface; artificial drainage may be difficult to install but is required for crop growth

____  7.	Water and air move freely, but slower than other soils

____  8.	Surface is wet for many weeks throughout the year; mottling is prominent just below the surface; artificial drainage usually needed

____  9.	Most often sandy or intermediately textured soils; color is uniform except near the deep water table, where it becomes mottled

Complete the following short answer questions.

10.	What are the steps in locating property in a soil survey manual?

11.	What is the difference between the permanent and temporary limitations used in determining a soil’s uses? Give an example of each.

Match the soil classification on the right with its description on the left.

a.	I

b.	II

c.	III

d.	IV

e.	V

f.	VI

g.	VII

h.	VIII

____ 12.	Requires moderate conservation practices

____ 13.	Mountainous areas

____ 14.	Prevents use of standard farm equipment due to limitations

____ 15.	Requires very careful management and/or special conservation practices

____ 16.	Very few limitations

____ 17.	Very severe limitations

____ 18.	Requires special conservation practices with extra costs and effort

____ 19.	Used for grassland, range, or wildlife due to severe limitations

List potential problems affecting plant growth for each of these factors.

20.	Erosion:

21.	Permeability:

22.	Soil texture: