IT ESSENTIALS V. 4.1

Module 3

Computer Assembly Step-by-Step

| 3.1 Open the Case |  |  |
| --- | --- | --- |
| 1. | What does form factor refer to? | Size and shape of the case |
| 2. | How should the workspace be prepared? | Adequate lighting, good ventilation, comfortable room temperature, workbench accessible from all sides |
| 3.2 Install the power supply |  |  |
| 3. | What are the power supply installation steps? | - Insert power supply into case - Align holes in power supply with holes in case - Secure power supply to case using proper screws |
| Virtual Desktop |  |  |
| Lab: Install Power Supply |  |  |
| 3.3 Attach the components to the motherboard and install the motherboard |  |  |
| 3.3.1 Install a CPU and a heat sink/fan assembly |  |  |
| 4. | How is the CPU secured to the socket on the motherboard? | With a locking assembly |
| 5. | What is thermal compound used for? | To help conduct heat away from the CPU |
| 6. | How do you clean a used CPU? | Isopropyl alcohol |
| 7. | What is the heat sink/fan assembly? | A two-part cooling device |
| 8. | What is the connection 1 indicator on the CPU lined up with on the CPU socket? | Pin 1 |
| 3.3.2 Install the RAM |  |  |
| 9. | What does RAM provide? | Temporary data storage for the CPU while the computer is operating |
| 10. | What is meant by volatile memory? | Its contents are lost when the computer is shut down |
| 3.3.3 Install the motherboard |  |  |
| 11. | What is used to mount the motherboard and to prevent it from touching he metal portions of the case? | Plastic and metal standoffs |
| Virtual Desktop |  |  |
| Lab: Install the Motherboard |  |  |
| 3.4 Install Internal Drives |  |  |
| 12. | What are the steps for installing a hard disk drive? | - Position the HDD so it aligns with the 3.5 inch drive bay - Insert the HDD in drive bay so screw hold in drive line up with screw holes in the case - Secure the HDD to case using proper screws |
| Virtual Desktop |  |  |
| 3.5 Install drives in external bays |  |  |
| 13. | What are external bays? | Allow access to the media without opening the case |
| 3.5.1 Install the optical drive |  |  |
| 14. | What is an optical drive? | A storage device that reads and writes information to CDs and DVDs |
| 15. | What kind of connector provides power to the optical drive from the power supply? | Molex |
| 16. | What kind of cable connects the optical drive to the motherboard? | PATA |
| 3.5.2 Install the Floppy Drive |  |  |
| 17. | What is a floppy disk drive (FDD)? | A storage device that reads and writes information to a floppy disk |
| 18. | What kind of power connector provides the FDD with power from the power supply? | berg |
| Virtual Desktop |  |  |
| Lab: Install the Drives |  |  |
| 3.6 Install adapter cards |  |  |
| 19. | Why would you use adapter cards? | To add functionality |
| 3.6.1 Install the NIC |  |  |
| 20. | Why do you need a NIC? | To connect to a network |
| 3.6.2 Install the wireless NIC |  |  |
| 21. | What type of expansion slots do wireless NICs use? | PC I & PCIe |
| 3.6.3 Install the video adapter card |  |  |
| 22. | What is a video adapter card? | The interface between a computer and a display monitor |
| 23. | What type of expansion slots do video adapter cards use? | PCI, AGP, PCIe |
| Virtual Desktop |  |  |
| Lab: Install Adapter Cards |  |  |
| 3.7 Connect All Internal Cables |  |  |
| 24. | What are power cables used for? | To distribute electricity from the power supply to the motherboard and other components |
| 25. | What do data cables do? | Transmit data between the motherboard and storage devices |
| 3.7.1 Connect the power cables |  |  |
| 26. | Haw many pins does the ATX main power connector have? | 20 or 24 |
| 27. | How many pins are on a SATA power connector? | 15 |
| 28. | What is a 4-pin berg connector used for? | Floppy drive |
| 3.7.2 Connect the data cable |  |  |
| 29. | What are the types of data cables? | PATA, SATA, floppy drive |
| 30. | What is another name for a PATA cable? | Ribbon cable |
| 31. | What does a stripe on the data cable indicate? | Location of pin 1 |
| 32. | Where is the pin 1 indicator on the drive connector? | Closest to the power connector on the drive |
| 33. | If a motherboard has two PATA drive controllers, how many PATA drive can it support? | 4 |
| 34. | What type of connector does a SATA data cable have? | 7 pin |
| 35. | What type of a connector does a floppy drive data cable have? | 34 pin |
| 36. | If the floppy drive activity light displays continuously, what is the problem? | Ribbon cable is not installed correctly |
| Virtual Desktop |  |  |
| Lab: Install Internal Cables |  |  |
| 3.8 Reattach the side panels and connect external cables to the computer |  |  |
| 3.8.1 Re-attach the side panels |  |  |
| 3.8.2 Connect external cables to the computer |  |  |
| 37. | Should you force a connector? | No |
| 38. | When should you plug in the power cable? | After you have connected all other cables |
| Virtual Desktop |  |  |
| Lab: Complete Computer Activity |  |  |
| 3.9 Boot the computer for the first time |  |  |
| 39. | When booting a computer, what performs a check on all of the internal components? | BIOS –basic input/output system |
| 40. | What does POST stand for? | Power-on Self-Test |
| 3.9.1 Identify Beep Codes |  |  |
| 41. | What does POST check for? | To see that all of the hardware in the computer is operating correctly |
| 42. | What does a single beep indicate? | The computer is functioning properly |
| 43. | What determines what the beep codes mean? | The BIOS manufacturer |
| 3.9.2 Describe BIOS setup |  |  |
| 44. | What is contained in BIOS? | Setup programs used to configure settings for hardware devices |
| 45. | What does CMOS stand for? | Complementary Metal Oxide Semiconductor |
| 46. | What key is typically pressed to enter the BIOS setup program? | DEL |
| LAB: Boot the Computer |  |  |