IT ESSENTIALS V. 4.1

Module 1

INTRODUCTION TO THE PERSONAL COMPUTER

| 1.0 Introduction |  |  |
| --- | --- | --- |
| 1. | What is information technology? | The design, development, implementation, support, and management of computer hardware and software applications |
| 1.1 Explain IT Industry Certifications |  |  |
| 1.1.1 Identify education and certifications |  |  |
| 1.1.2 Describe the A+ Certification |  |  |
| 2. | Who developed the A+ certification program? | CompTIA |
| 3. | What does A+ certification signify? | That a candidate is a qualified PC hardware and software technician |
| 4. | What does the A+ Essentials exam measure? | It tests for the fundamentals of computer technology, networking and security, communication skills and professionalism required of all entry-level IT professionals. |
| Worksheet – Job Opportunities |  |  |
| 1.1.3 Describe the EUCIP certificate |  |  |
| 5. | What are the five modules of the EUCIP IT Administrator Certificate? | PC hardware, operating systems, local area network and network services, expert network use and IT security |
| 1.2 Describe a Computer System |  |  |
| 6. | What makes up a computer system? | Hardware and software components |
| 7. | What is hardware? | Physical equipment |
| 8. | What are examples of hardware? | Case, storage drives, keyboards, monitors, cables, speakers, printers |
| 9. | What is software? | The operating system and programs |
| 10. | What does the operating system do? | Instructs the computer how to operate |
| 1.3 Identify the names, purposes, and characteristics of cases and power supplies |  |  |
| 11. | What does the computer case do? | Provides protection and support for the internal components of the computer |
| 12. | What determines the size and shape of the computer case? | The motherboard an internal components |
| 13. | How large should the compute case be? | Durable, easy to service, enough room for expansion |
| 14. | How large should the power supply be? | Large enough to supply enough power for the components that are currently installed and allow additional components to be added at a later time |
| 1.3.1 Describe cases |  |  |
| 15. | What does a computer case contain? | The framework to support the internal components of a computer while provided an enclosure for added protection |
| 16. | What are computer cases made of? | Plastic, steel, and aluminum |
| 17. | What are the size and layout of a case called? | Form factor |
| 18. | What are the basic form factors for computer cases? | Desktop, tower |
| 19. | What factors must be considered when choosing a case? | Size of motherboard, number of external or internal drive locations, available space |
| 20. | How are internal components grounded? | Attaching to the case |
| 1.3.2 Describe Power Supplies |  |  |
| 21. | What does a power supply do? | Converts AC power coming from the wall into DC power |
| 22. | What can protect a computer from problems caused by a change in power? | UPS – Uninterruptible Power Supply |
| 23. | What does a power invertor do? | Provides AC power to the computer rom a built-in battery by converting DC current of the UPS battery into AC power |
| 24. | What is a keyed connector? | Designed to be inserted in only one direction |
| 25. | What is a molex connector used to connect? | An optical drive or a hard drive |
| 26. | What does a berg connector connect? | A floppy drive |
| 27. | What kind of connection is used to connect the motherboard? | 20 or 24 pin slotted connector |
| 28. | What is the voltage of the yellow wire? | +12 V |
| 29. | What is the voltage of the blue wire? | -12 V |
| 30. | What is the voltage of the orange wire? | +3.3 V |
| 31. | What is the voltage of the red wire? | +5 V |
| 32. | What is the voltage of the white wire? | -5 V |
| 33. | What color is the ground wire? | black |
| 34. | What are the four basic units of electricity? | Voltage (V), current (I), Power (P), Resistance (R) |
| 35. | What is voltage? | A measure of the force required to push electrons through a circuit |
| 36. | What is current? | A measure of the amount of electrons going through a circuit |
| 37. | How is current measured? | In amps |
| 38. | What is power? | A measure of the pressure required to push electrons through a current (voltage) multiplied by the number of electrons going through that circuit (current) |
| 39. | What is the measurement of power? | Watts |
| 40. | What is resistance? | the opposition to the flow or current in a circuit |
| 41. | How is resistance measured? | Ohms |
| 42. | Lower resistance allows _______ current | More or Less (circle the correct answer) |
| 43. | What is Ohm’s Law? | V=IR |
| 44. | What will result in higher power? | Increasing the current or voltage |
| 1.4 Identify the names, purposes, and characteristics of internal components |  |  |
| 1.4.1 Identify the names, purposes, and characteristics of motherboards |  |  |
| 45. | What is the motherboard? | The main printed circuit board that contains the buses |
| 46. | What is a bus? | Electrical pathway |
| 47. | What are other names for the motherboard? | System board, backplane, or mainboard |
| 48. | What items are on the motherboard? | CPU, RAM, expansion slots, heat sink/fan assembly, BIOS chip, chipset and embedded wires |
| 49. | What does form factor refer to? | Size and shape of the board |
| 50. | What does the form factor determine? | How individual components attach to the motherboard and the shape of the computer case |
| 51. | What is the most common form factor in desktop computers? | AT |
| 52. | What is the newer motherboard form factor? | ATX |
| 53. | What is the chipset? | Integrated circuits attached to the motherboard that control how system hardware interacts with the CPU and motherboard |
| 54. | What determines the type of CPU that can be installed? | The socket on the motherboard |
| 55. | What are the two components that make up the chipset? | Northbridge and Southbridge |
| 56. | Which component controls access to the Ram, video card and the speeds at which the CPU can communicate with them? | Northbridge |
| 57. | Which component allows the CPU to communicate with the hard drives, sound card, USP ports and other I/O ports? | Southbridge |
| 1.4.2 Identify the names, purposes, and characteristics of CPUs |  |  |
| 58. | What is the brain of the computer? | CPU |
| 59. | Who are the most common CPU manufacturers? | Intel, AMD |
| 60. | What is the connector that interfaces between the motherboard and the processor? | CPU socket or slot |
| 61. | What is PGA? | Pin Grid Array |
| 62. | What is ZIF? | Zero Insertion Force |
| 63. | What does ZIF refer to? | The amount of force needed to install a CPU into the motherboard socket or slot |
| 64. | What is PGA architecture? | The pins on the underside of the processor are inserted into the socket |
| 65. | What is a program? | A sequence of stored instructions |
| 66. | What are the two major CPU architecture instruction sets? | RISC and CISC |
| 67. | What does RISC stand for? | Reduced Instruction Set Computer |
| 68. | What does CISC stand for? | Complex Instruction Set Computer |
| 69. | Which architecture takes fewer steps per operation? | CISC |
| 70. | What is hyperthreading? | The CPU has multiple pieces of code being executed simultaneously on each pipeline |
| 71. | How is the power of a CPU measured? | By the speed and amount of data that it can process |
| 72. | How the speed of a CPU rated? | Cycles per second; ie MHz or GHz |
| 73. | What determines the amount of data that a CPU can process at one time? | The size of the processor data bus |
| 74. | The wider the processor bus, the more ______ the processor. | Powerful |
| 75. | What size processor data bus do current processors have? | 32 bit or 64 bit |
| 76. | What is overclocking? | A technique used to make a processor work at a faster speed than its original specifications |
| 77. | Is overclocking a reliable way to improve computer performance? | No |
| 78. | What is CPU throttling? | When the processors runs at less than the rated speed to conserve power or produce less heat |
| 79. | What is MMX? | A set of multimedia instructions built into Intel processors |
| 80. | What has replaced MMX? | SSE – Streaming Single-Instruction-Multi-date Extensions |
| 81. | What is a single core CPU? | One core inside a single CPU that handle all of the processing capability |
| 82. | What are the most common multiple core processors? | Dual Core, Triple Core, and Quad Core |
| 83. | Why would you use a Quad Core CPU? | Enhanced software applications |
| 1.4.3 Identify the names, purposes, and characteristics of Cooling Systems |  |  |
| 84. | What causes heat? | The flow of current within the components |
| 85. | How can you remove heat? | Increasing air flow |
| 86. | What makes the cooling process more efficient? | A case fan |
| 87. | How does a water-cooling system work? | A metal plate is placed over the processor and water is pumped over the top to collect the heat the CPU creates |
| 1.4.4.Identify the names, purposes, and characteristics of ROM and RAM |  |  |
| 88. | What does ROM stand for? | Read Only Memory |
| 89. | What is stored in ROM? | Basic instructions for booting the computer and loading the operating system |
| 90. | What is another name for ROM? | Firmware |
| 91. | Do ROM chips retain their contents when the computer is powered down? | Yes |
| 92. | What are the different types of ROM? | PROM, EPROM, EEPROM |
| 93. | Can a PROM chip be erased or rewritten? | No |
| 94. | Which h type of PROM can be erased and rewritten without having the remove the chip from the computer? | EEPROM |
| 95. | What does RAM stand for? | Random Access Memory |
| 96. | What is meant by volatile memory? | The contents are erased when the computer is powered off |
| 97. | Why would you want to have more RAM in a computer? | The more capacity the computer has to hold and process large programs and files, to enhance system performance |
| 98. | What are the different types of RAM? | DRAM, SRAM, FPM Memory, EDO memory, SDRAM, DDR SDRAM, DDR2, DDR3, RDRAM |
| 99. | Which type of RAM speeds up the access time to retrieve data from memory because the CPU does not have to wait for one data access cycle to end before another begins? | EDO memory |
| 100. | What are the different types of memory modules? | DIP, SIMM, DIMM, RIMM, SODIMM |
| 101. | What are the two configurations of SIMM chips? | 30 pin and 72 pin |
| 102. | What type of DIMM chips are there? | 168 pin SDRAM, 184 pin DDR, 240 pin DDR2 |
| 103. | What is the configuration of the RIMM modules? | 184 pin |
| 104. | What is SODIMM used for? | Laptops, printers, or devices where conserving space is desirable |
| 105. | What does the speed of memory impact? | How much data a processor can process because faster memory improve the performance of the processor |
| 106. | What does DDR technology do? | Doubles the maximum bandwidth of SDRAM |
| 107. | What is cache memory used for? | To store the most frequently used data |
| 108. | What are the three types of cache memory? | L1, L2, L3 |
| 109. | When do memory errors occur? | When the data is not stored correctly in the RAM chips |
| 110. | What are the three methods of error checking? | Non-parity, parity, ECC |
| 1.4.5 Identify the names, purposes, and characteristics of adapter cards |  |  |
| 111. | What do adapter cards do? | Increase the functionality of a computer by adding controllers for specific devices or by replacing malfunctioning ports |
| 112. | Why would you use adapter cards? | To expand and customize the capability of the computer |
| 113. | What does SCSI stand for? | Small Computer System Interface |
| 114. | What does RAID stand for? | Redundant Array of Independent Disks |
| 115. | What does USB stand for? | Universal Serial Bus |
| 116. | What expansion slot is used for video adapters? | AGP |
| 117. | What expansion slot is an IBM proprietary 32-bit expansion slot? | MCA |
| 118. | What was used in computer systems with e LPX form factor to allow adapter cards to be installed horizontally? | Riser card |
| 1.4.6 Identify the names, purposes, and characteristics of storage drives |  |  |
| 119. | What do storage drives do? | Read or write information to magnetic or optical storage media |
| 120. | What are some common types of storage devices? | Floppy drive, hard drive, optical drive, flash drive |
| 121. | What does a floppy drive used for storage? | A removable 3.5 inch floppy disk |
| 122. | What is the storage capacity of a floppy disk? | 720 KB or 1.44 MB |
| 123. | What does the hard drive contain? | The operating system and applications |
| 124. | How is the storage capacity of a hard drive measured? | Billions of bytes or gigabytes (GB) |
| 125. | How is the speed of a hard drive measured? | Revolutions per minutes (RPM) |
| 126. | How do magnetic hard drives spin magnetic platters and drive heads? | With a drive motor |
| 127. | Do solid state drives (SSDs) have moving parts? | No |
| 128. | How do optical drives read data on optical media? | With lasers |
| 129. | What are three types of optical drives? | Compact Disc (CD), Digital Versatile Disc (DVD), Blu-Ray Disc (BD) |
| 130. | What is the storage capacity of a CD? | 720 MB |
| 131. | What is the storage capacity of a DVD? | 4.3 GB single-layer 8.5 GB dual-layer |
| 132. | What is the storage capacity of a BD? | 25 GB single-layer 50 GB dual-layer |
| 133. | Which optical media is a CD that can be recorded, erased, and re-recorded? | CD-RW |
| 134. | Which optical media is a DVD that can be recorded one time? | DVD +/- R |
| 135. | Which optical media contains pre-recorded movies, games, or software? | BD-ROM |
| 136. | Which optical media can record HD video and PC data storage one time? | BD-R |
| 137. | What is another name for an external flash drive? | Thumb drive, jump drive |
| 138. | Which type of hard drive interface uses a 40-pin connector? | IDE and EIDE |
| 139. | What size connector does the SATA interface use? | 7 pin data connection |
| 140. | Which cable can be up to 2 meters in length? | eSATA |
| 141. | How many devices can a SCSI controller connect? | 15 |
| 142. | What type of connector do SCSI interfaces use? | 50-pin, 68-pin, or 80-pin connector |
| 143. | What is RAID used for? | Provides a way to store data across multiple hard disks for redundancy |
| 144. | What are the terms that describe how RAID stores data on various disks? | Parity, striping, mirroring |
| 145. | What is the advantage of RAID 0? | Highest performance |
| 146. | What are the advantages of RAID 5? | Supports multiple simultaneous reads and writes; data is written across all drive with parity, data can be rebuilt from information found on other drives |
| 147. | How many drive are required for RAID 5? | Minimum of 3 |
| 1.4.7 Identify the names, purposes, and characteristics of internal cables |  |  |
| 148. | What kind of power connector is needed for PATA drives? | Molex |
| 149. | What kind of power connector is needed for floppy drive? | Berg 4-pin |
| 150. | How many types of SCSI data cables are there? | 3 |
| 151. | What are the types of SCSI data cables? | Narrow – 50-pin connector Wide – 68-pin connector Alt4 – 80-pin connector |
| 152. | What does a colored stripe on a cable indicate? | Pin 1 |
| Worksheet: Research Computer Components |  |  |
| 1.5 Identify the names, purposes, and characteristics of ports and cables |  |  |
| 153. | Describe a serial port. | DB9 or DB25 male connector |
| 154. | What is the maximum length of a serial cable? | 50 ft |
| 155. | What type of connector is used for a telephone cable? | RJ11 |
| 156. | How many devices can a single USB port in a computer support? | 127 |
| 157. | What is the transmission speed of USB 2.0? | 480 Mbps |
| 158. | What is Firewire? | A high-speed, hot-swappable interface that connects peripheral devices to a computer |
| 159. | How many devices can a single Firewire port support? | 63 |
| 160. | What standard does Firewire us? | IEEE 1394 |
| 161. | What is the difference between 1394A and 1394B? | 1394 A – data rate 400 Mbps, cable length 15 ft, 4 or 6 pin connector 1394 B – greater range of connections including Cat 5 and fiber; 3.2 Gbps |
| 162. | What type of parallel connector is used for a printer? | Type B 36-pin Centronics |
| 163. | How much data can a parallel cable transport at one time? | 8 bits |
| 164. | What is the standard for parallel cable? | IEEE 1284 |
| 165. | What is the maximum length of a parallel cable? | 15 feet |
| 166. | If a single SCSI device is connected to a SCSI port, how long can the cable be? | 80 feet |
| 167. | Where should SCSI devices be terminated? | At the endpoints of the chain |
| 168. | What is another name for a network port? | RJ45 |
| 169. | What is the maximum length of a network cable? | 328 feet |
| 170. | What does a PS/2 port connect? | Mouse, keyboard |
| 171. | What kind of a connector does a PS/2 port use? | 6-pin mini-din female connector |
| 172. | What are three common video ports? | S-video, VGA, DVI |
| 173. | Which video port provides analog output to a monitor? | VGA |
| 174. | Which video port provides an uncompressed digital output to a monitor? | DVI |
| 175. | How many cables does a RGB cable have? | 3 (red, green, blue) |
| 1.6 Identify the names, purposes, and characteristics of input devices |  |  |
| 176. | What is an input device used for? | To enter data or instructions into a computer |
| 177. | What are the two most commonly used input devices? | Mouse, keyboard |
| 178. | What is a hardware device that can be used to control more than one computer using a single keyboard, monitor, and mouse | KVM Switch (Keyboard, Video, Mouse) |
| 179. | What is used for biometric identification? | A feature that is unique to an individual user such as a fingerprint, retinal scanner, or voice recognition |
| 180. | What causes a touch screen to work? | Pressure-sensitive transparent panel |
| 181. | What does a scanner do? | Digitizes an image or document |
| 182. | What type of scanner reads UPC bar codes? | Bar code reader |
| 1.7 Identify the names, purposes, and characteristics of output devices |  |  |
| 183. | What is an output device used for? | To present information to the user from a computer |
| 184. | What are the primary output devices for a computer? | Monitors and projectors, printers |
| 185. | What are the three types of monitors? | LCD, CRT, Projector |
| 186. | What is the important difference between monitor types? | The technology used to create an image |
| 187. | How is the image created on the screen in CRT technology? | The combination of glowing and non-glowing areas |
| 188. | Which technology is commonly used in flat panel monitors and laptops? | LCD |
| 189. | What is the difference between active matrix and passive matrix? | Active matrix allows each pixel to be controlled which creates very sharp color images |
| 190. | How does DLP technology work? | A spinning color wheel works with a micromirror device; each mirror corresponds to a specific pixel; each mirror reflects light toward or away from the projector optics |
| 191. | What does monitor resolution refer to? | The level of image detail that can be reproduced |
| 192. | What is a pixel? | Picture element; tiny dots that comprise a screen |
| 193. | What is dot pitch? | The distance between pixels on the screen |
| 194. | Which dot pitch produces a better image? | A lower dot pitch number |
| 195. | What is the contrast ratio? | A measurement of the difference in intensity of light between the brightest point and the darkest point |
| 196. | What is the refresh rate? | How often per second the image is rebuilt |
| 197. | Does a higher or lower refresh rate produce a better image? | higher |
| 198. | What is the horizontal resolution? | The number of pixels in a line |
| 199. | What is the vertical resolution? | Number of lines in a screen |
| 200. | What is color resolution? | The number of colors that can be reproduced |
| 201. | What is aspect ratio? | The horizontal to vertical measurement of the viewing area of a monitor |
| 202. | What is native resolution? | The number of pixels that a monitor has |
| 203. | What is a printer? | An output device that creates hard copies of computer files |
| 204. | What is an all-in-one type printer? | Provides multiple services such as printing, scanning faxing, and copying |
| 1.8 Explain System Resources and Their Purposes |  |  |
| 205. | What are the three common resources? | IRQ, I/O Port Addresses, DMA |
| 206. | What does IRQ stand for? | Interrupt Request |
| 207. | What does DMA stand for? | Direct Memory Access |
| 208. | What is an IRQ used for? | For computer components to request information from the CPU |
| 209. | How man IRQS are there? | 16 |
| 210. | How is the priority of the request determined? | By the IRQ number |
| 211. | Fill in the IRQ Chart | 0 - System Timer 1 – Keyboard Controller 2 – 2nd IRQ Controller Cascade 3 – Serial 2 (Com 2) 4 – Serial 1 (Com 1) 5 – Serial/Parallel 2 (LPT2) 6 – Floppy Drive 7 – Parallel 1 (LPT1) 8 – Real-Time Clock 9 - Available 10 - Available 11 - Available 12 – Mouse Port/Available 13 – Math Co-Processor 14 – Primary IDE 15 – Secondary IDE |
| 212. | What are I/O port addresses used for? | To communication between devices and software |
| 213. | How many I/O ports are in a computer? | 65,535 |
| 214. | Fill in the I/O Port Chart | Com 1 – 3f8 Com 2 – 2f8 Com 3 – 3e8 Com 4 – 2e8 LPT1 – 378 LPT2 – 278 |
| 215. | What devices use DMA channels? | High-speed devices |
| 216. | How many DMA channels are there? | 8 |
| 217. | Fill in the DMA Channel Chart | 0 – Sound 1 – Sound 2 - Floppy Drive Controller 3 – LPT 1 in ECP mode 4 – Cascade for DMA 0-3 5 – Sound 6 - Available 7 – Available |